RIVANNA RIVER STEWARD REPORT

River Stewards: Dan Triman    Date:  5-15-16

Starting Location and Time: Darden Towe (1:00 PM)

Ending Location and Time:  Riverview Park (2:30 PM)

Weather:  Cloudy, Windy   Recent Precipitation: Some rain recently

USGS Gauges: Earlysville 2.10 ft (129 ft3 / sec), Palmyra 3.86 ft (787 ft3 / sec),

Water Color: Mostly Clear   Water Odor: None    Air Temp. : 60 deg F

DO: 4 ppm       PH: 8     Turbidity: 10 JTU      Water Temp. : 18 deg C

Water Testing Location: Darden Towe Boat Ramp River Left

---------------------------------------------------------------------------------

Land Use:

Residential mainly, with some commercial use and park property

Wildlife Sightings and Plant Life:

Turtles, Crows, Cardinals, Fish, Geese, Great Blue Heron, smaller birds. Several patches of dense bamboo near Darden. Vegetation is much fuller now than a few months ago. Turtles were abundant on the River today, and there was at least 3 families of adult Geese and their babies spotted.

People Interactions:

There were a couple of people fishing along the river, as well as 2 other people kayaking the Rivanna today. Numerous other people walking along the trails alongside the River.

Litter and Pollution:

There was not much trash found today. Just a couple of plastic drink bottles, and some fishing bait boxes. There was 1 tire buried in the River noticed as well. Overall the River looked pretty clean today.

Overall Summary:

Water levels are still up slightly, but have come down since the last week or two of heavy rains. The Riverview Park steps still need attention as the soil has eroded at the bottom (see photo below). There was also a pipe that is just hanging out of the water near Darden Towe Park (River Left – see photo below).

Actions Needed – level of priority (1 highest/5 lowest):

4 – Clean sediment off Darden Towe boat ramp and Riverview steps; 4 – Determine if the pipe near Darden Towe needs to be fixed; 2 – fix Riverview Park steps for safety issues

Figure 1: Pipe hanging in water near Darden Towe                     Figure 2: Patch of bamboo just down river from Darden Towe

Figure 3: Some of the many turtles seen today                            Figure 4: 2 Kayakers heading towards Free Bridge

Figure 5: Buried tire in the River                                                       Figure 6: Heavy eroded bank

Figure 7: One of the families of Geese                                           Figure 8: Pink flowers blooming

Figure 9: Riverview steps still in need of repair